A-001-03-01

What is the resonant frequency of a series R-L-C circuit if R is 47 ohms, L is

50 microhenrys and C is 40 picofarads?

1 - 1.78 MHz

2 - 3.56 MHz

3 - 7.96 MHz

4 - 79.6 MHz

A-001-03-02

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 40 microhenrys and C is 200 picofarads?

1 - 1.99 kHz

2 - 1.99 MHz

3 - 1.78 kHz

4 - 1.78 MHz

A-001-03-03

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 50 microhenrys and C is 10 picofarads?

1 - 7.12 kHz

2 - 3.18 MHz

3 - 3.18 kHz

4 - 7.12 MHz

A-001-03-04

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 25 microhenrys and C is 10 picofarads?

1 - 63.7 MHz

2 - 10.1 kHz

3 - 63.7 kHz

4 - 10.1 MHz

A-001-03-05

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 3 microhenrys and C is 40 picofarads?

1 - 13.1 MHz

2 - 14.5 MHz

3 - 13.1 kHz

4 - 14.5 kHz

A-001-03-06

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 4 microhenrys and C is 20 picofarads?

1 - 19.9 MHz

2 - 17.8 MHz

3 - 19.9 kHz

4 - 17.8 kHz

A-001-03-07

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 8 microhenrys and C is 7 picofarads?

1 - 28.4 MHz

2 - 21.3 MHz

3 - 2.84 MHz

4 - 2.13 MHz

A-001-03-08

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 3 microhenrys and C is 15 picofarads?

1 - 35.4 MHz

2 - 23.7 MHz

3 - 35.4 kHz

4 - 23.7 kHz

A-001-03-09

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 4 microhenrys and C is 8 picofarads?

1 - 49.7 MHz

2 - 28.1 MHz

3 - 49.7 kHz

4 - 28.1 kHz

A-001-03-10

What is the resonant frequency of a series R-L-C circuit, if R is 47 ohms, L

is 1 microhenry and C is 9 picofarads?

1 - 53.1 MHz

2 - 5.31 MHz

3 - 17.7 MHz

4 - 1.77 MHz

A-001-03-11

What is the value of capacitance (C) in a series R- L-C circuit, if the circuit

resonant frequency is 14.25 MHz and L is 2.84 microhenrys?

1 - 2.2 microfarads

2 - 44 microfarads

3 - 44 picofarads

4 - 2.2 picofarads