A-001-04-01

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 1 microhenry and C is 10 picofarads?

1 - 15.9 kHz

2 - 50.3 MHz

3 - 50.3 kHz

4 - 15.9 MHz

A-001-04-02

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 2 microhenrys and C is 15 picofarads?

1 - 29.1 MHz

2 - 29.1 kHz

3 - 5.31 MHz

4 - 5.31 kHz

A-001-04-03

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 5 microhenrys and C is 9 picofarads?

1 - 23.7 kHz

2 - 3.54 MHz

3 - 3.54 kHz

4 - 23.7 MHz

A-001-04-04

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 2 microhenrys and C is 30 picofarads?

1 - 2.65 MHz

2 - 20.5 MHz

3 - 2.65 kHz

4 - 20.5 kHz

A-001-04-05

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 15 microhenrys and C is 5 picofarads?

1 - 2.12 kHz

2 - 2.12 MHz

3 - 18.4 MHz

4 - 18.4 kHz

A-001-04-06

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 3 microhenrys and C is 40 picofarads?

1 - 1.33 kHz

2 - 1.33 MHz

3 - 14.5 MHz

4 - 14.5 kHz

A-001-04-07

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 40 microhenrys and C is 6 picofarads?

1 - 6.63 MHz

2 -10.3 MHz

3 - 6.63 kHz

4 - 10.3 kHz

A-001-04-08

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 10 microhenrys and C is 50 picofarads?

1 - 7.12 MHz

2 - 7.12 kHz

3 - 3.18 MHz

4 - 3.18 kHz

A-001-04-09

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 200 microhenrys and C is 10 picofarads?

1 - 3.56 kHz

2 - 7.96 MHz

3 - 7.96 kHz

4 - 3.56 MHz

A-001-04-10

What is the resonant frequency of a parallel R-L-C circuit if R is 4.7

kilohms, L is 90 microhenrys and C is 100 picofarads?

1 - 1.77 kHz

2 - 1.77 MHz

3 - 1.68 MHz

4 - 1.68 kHz

A-001-04-11

What is the value of inductance (L) in a parallel R-L-C circuit, if the resonant

frequency is 14.25 MHz and C is 44 picofarads?

1 - 253.8 millihenrys

2 - 3.9 millihenrys

3 - 0.353 microhenry

4 - 2.8 microhenrys